Multi-User Activity: ACL Solution
Solutions can vary. One possible solution for Peer0:

Router0:

hostname Router

!

!

!

!

!

ip ssh version 1

!

!

interface FastEthernet0/0

 ip address 10.1.4.1 255.255.255.0

 duplex auto

 speed auto

!

interface FastEthernet0/1

 no ip address

 duplex auto

 speed auto

 shutdown

!

interface Serial0/0

 ip address 10.1.5.2 255.255.255.0

 ip access-group 101 in

!

interface Serial0/1

 ip address 10.1.3.2 255.255.255.0

 clock rate 56000

!

interface Serial0/2

 no ip address

 shutdown

!

interface Serial0/3

 no ip address

 shutdown

!

interface FastEthernet1/0

 no ip address

 duplex auto

 speed auto

 shutdown

!

interface FastEthernet1/1

 no ip address

 duplex auto

 speed auto

 shutdown

!

router eigrp 101

 network 10.0.0.0

 auto-summary

!

ip classless

!

!

access-list 101 permit ip host 10.100.1.1 any

access-list 101 deny tcp any any eq telnet

access-list 101 deny icmp any any echo

access-list 101 permit tcp any any established

access-list 101 permit tcp any host 10.1.4.3 eq www

access-list 101 permit tcp 10.10.2.0 0.0.0.255 host 10.1.1.2 eq www

access-list 101 permit tcp 10.20.2.0 0.0.0.255 host 10.1.1.2 eq www

! assuming that peer1 and peer2 users are also participating

access-list 101 permit eigrp any any

!

!

!

no cdp run

!

line con 0

line vty 0 4

 login

!

!

end

Router#
Router1:

Current configuration : 1188 bytes

!

version 12.2

no service password-encryption

!

hostname Router

!

!

enable secret 5 $1$mERr$hx5rVt7rPNoS4wqbXKX7m0

!

!

!

!

ip ssh version 1

!

!

interface FastEthernet0/0

 ip address 10.1.1.1 255.255.255.0

 ip access-group 101 out

 duplex auto

 speed auto

!

interface FastEthernet0/1

 ip address 10.1.2.1 255.255.255.0

 duplex auto

 speed auto

!

interface Serial0/0

 ip address 10.1.3.1 255.255.255.0

!

interface Serial0/1

 no ip address

 shutdown

!

interface Serial0/2

 no ip address

 shutdown

!

interface Serial0/3

 no ip address

 shutdown

!

interface FastEthernet1/0

 no ip address

 duplex auto

 speed auto

 shutdown

!

interface FastEthernet1/1

 no ip address

 duplex auto

 speed auto

 shutdown

!

router eigrp 101

 network 10.0.0.0

 auto-summary

!

ip classless

!

!

access-list 101 permit ip host 10.100.1.1 any

access-list 101 permit tcp 10.10.2.0 0.0.0.255 host 10.1.1.2 eq www

access-list 101 permit tcp 10.20.2.0 0.0.0.255 host 10.1.1.2 eq www

! assuming that peer1 and peer2 users are also participating

access-list 101 permit ip 10.1.2.0 0.0.0.255 host 10.1.1.2

access-list 101 permit ip host 10.1.4.2 host 10.1.1.2

access-list 101 permit tcp 10.1.4.0 0.0.0.255 host 10.1.1.2 eq www

!

!

!

no cdp run

!

line con 0

line vty 0 4

 login

!

!

end
